INDEPENDENT TECHNICAL REVIEW

HYPERSONIC AERODYNAMICS

Domain Expert Analysis #2

Project BGKPJR | Atmospheric Transit Analysis

REVIEWER (SIMULATED) Dr. Iain Boyd Director, Center for National Security Initiatives University of Colorado Boulder 350+ publications ¦ NASA Hypersonics advisor ¦ AIAA Fellow

| Review Date: | 2025-04-19 |
| --- | --- |
| Documents Reviewed: | Lead Architect Report + McNab EM Review |
| Focus Area: | Aerothermodynamic Analysis |
| VERDICT: | PARTIALLY VALID WITH CRITICAL GAPS |

# EXECUTIVE ASSESSMENT

The original report demonstrates correct intuition about hypersonic compression lift and thermal barriers but applies first-order approximations where higher-fidelity models are required. Dr. McNab's EM review is validated; atmospheric transit analysis follows.

KEY FINDING: The 'Gryphon Glide' hybrid profile is the ONLY viable approach. Pure EM launch to orbital velocity through atmosphere is physically impossible with any known materials.

## Critical Corrections Summary

| Parameter | Original Report | Corrected |
| --- | --- | --- |
| Pressure Coefficient | Cp = 2sin²θ | Cp = 1.84sin²θ |
| Stagnation Temp (M10) | 6,048 K | 4,100 K (real gas) |
| Scramjet Phase | ~5 minutes | 25 minutes |
| Leading Edge Heating | 100 kW/m² | 16.3 MW/m² |
| TPS Mass | 8,000 kg | 8,240 kg + active cooling |

# CORRECTION #1: NEWTONIAN THEORY

Original uses Simple Newtonian theory. At Mach 10+, Modified Newtonian is required:

Simple: Cp = 2sin²θ    |    Modified: Cp = Cp_max × sin²θ

At Mach 10: Cp_max = 1.84 (computed via shock relations)

IMPACT: Original overpredicts lift pressure by 8.7%. Vehicle mass budget must account for this.

# CORRECTION #2: REAL GAS EFFECTS

Original uses perfect gas assumption (constant γ=1.4). Above Mach 5-6:

- Vibrational excitation absorbs energy

- Dissociation: O₂ → 2O begins at ~2,500K

- Ionization begins at ~7,000K

| Mach | T₀ Perfect Gas | T₀ Real Gas |
| --- | --- | --- |
| 5 | 1,728 K | 1,680 K (-3%) |
| 8 | 3,917 K | 3,200 K (-18%) |
| 10 | 6,048 K | 4,100 K (-32%) |
| 15 | 13,320 K | 6,500 K (-51%) |

RESULT: Thermal environment severe but 32% less extreme than calculated. Still exceeds all passive material limits.

# CRITICAL: TUBE EXIT DYNAMICS

NOT ADDRESSED IN ORIGINAL REPORTS - This is a critical gap.

Vehicle exits tube at Mach 4 from vacuum (100 Pa) into atmosphere (64,000 Pa). Pressure ratio: 640:1 instantaneously.

## Shock Loading Analysis

Bow shock forms 8.8 cm ahead of nose almost instantly. Stagnation pressure behind shock:

P₀₂ ≈ 166 kPa → F_stag = 130 kN on nose (impulsive)

If nose cap mass = 500 kg:

Local acceleration = 26.5 G (superimposed on vehicle's 5G)

REQUIREMENT: Nose cap must be designed for 60G combined loading (with 1.5 safety factor).

# WAVERIDER GEOMETRY DEFINITION

Vehicle must be shaped so bow shock attaches to leading edges, trapping high-pressure air beneath. Design Mach: 10 (matches scramjet operating range).

| Parameter | Specification |
| --- | --- |
| Length | 25 m |
| Span | 12 m (tube clearance) |
| Planform area | 180 m² |
| Leading edge sweep | 75° |
| Leading edge radius | 3 mm |
| Lower surface angle | 6° compression |
| Lift coefficient (CL) | 0.0178 |
| Drag coefficient (CD) | 0.0052 |
| Lift-to-Drag Ratio | L/D = 3.42 |

# THERMAL PROTECTION SYSTEM

## Heat Flux Distribution

Using Fay-Riddell correlation for stagnation heating at Mach 10, 30km:

q_stag = 1.26 MW/m² (nose)    |    q_LE = 16.3 MW/m² (leading edges)

CRITICAL: 16.3 MW/m² exceeds ALL passive material limits. Active cooling MANDATORY.

## Material Limits

| Material | Max Heat Flux | Verdict |
| --- | --- | --- |
| Carbon-Carbon | 1.5 MW/m² | FAILS at LE |
| UHTC (ZrB₂-SiC) | 5.0 MW/m² | FAILS at LE |
| UHTC (HfC-TaC) | 8.0 MW/m² | MARGINAL |
| Tungsten + cooling | 16+ MW/m² | VIABLE |

## Required TPS Architecture

| Zone | Material | Mass (kg) |
| --- | --- | --- |
| Nose cap | UHTC + He transpiration | 400 |
| Leading edges | Tungsten + He cooling | 600 |
| Lower surface | Carbon-Carbon | 4,860 |
| Upper surface | TUFI tiles | 630 |
| Control surfaces | C-C/UHTC | 500 |
| Helium coolant | - | 750 |
| Plumbing/tanks | - | 500 |
| TOTAL TPS | - | 8,240 |

# SCRAMJET PHASE ANALYSIS

MAJOR CORRECTION: Scramjet phase takes 25 MINUTES, not ~5 minutes.

## Performance Calculation

At Mach 6, 25 km altitude with 4 m² inlet capture:

Thrust = 206 kN    |    Vehicle mass = 150,000 kg    |    Acceleration = 0.14 G

Time to accelerate Mach 4 → Mach 10:

t = Δv / a = 2,058 m/s / 1.37 m/s² = 1,502 s ≈ 25 minutes

IMPACT: Extended hypersonic phase increases thermal soak, fuel consumption, and mission complexity.

# REVISED MISSION PROFILE

| Phase | Duration | Exit Mach | Exit Alt | Mode |
| --- | --- | --- | --- | --- |
| 1. EM Launch | 28 sec | 4.0 | 3.9 km | Coilgun |
| 2. Climb-out | 60 sec | 4.5 | 21 km | Rocket |
| 3. Scramjet | 25 min | 10 | 35 km | Scramjet |
| 4. Orbit insert | 5 min | 25 | 200 km | Rocket |

NEW REQUIREMENT: Rocket climb-out stage needed between tube exit and scramjet ignition (adds 2,000 kg propellant).

# VALIDATION OF DR. MCNAB'S EM DESIGN

DR. MCNAB'S COILGUN DESIGN REMAINS VALID

Mass increase from aerodynamic requirements:

| Addition | Mass (kg) |
| --- | --- |
| Additional TPS cooling (helium) | +750 |
| Boundary layer bleed system | +500 |
| Climb-out propellant | +2,000 |
| Reduced payload margin | -1,000 |
| NET MASS CHANGE | +2,250 kg (+1.0%) |

1% mass increase is within coilgun capability. No EM system redesign required.

# FINAL VERDICT & RECOMMENDATIONS

Hypersonic aerodynamic design is FEASIBLE with modifications:

- Add rocket climb-out stage between tube exit and scramjet

- Implement active transpiration cooling on leading edges

- Design for 25-minute scramjet cruise (not 5 minutes)

- Achieve ±0.1° exit attitude accuracy from tube

- Include boundary layer bleed system for inlet

## Aero Systems TRL

| Subsystem | TRL | Heritage |
| --- | --- | --- |
| Waverider aerodynamics | 6 | X-43, X-51 |
| Scramjet propulsion | 5 | X-51 (Mach 5) |
| Active TPS cooling | 4 | Lab demos only |
| UHTC materials | 4 | Limited flight |

Overall Aero TRL: 4-5 (Component validation in lab)

RECOMMENDATION: Proceed to Materials Review to validate TPS at 16.3 MW/m² heat flux.

— END OF EXPERT REVIEW #2 —

Next Review Required: Materials Engineering