TECHNICAL ANALYSIS REPORT

PROJECT BGKPJR

Hypersonic Electromagnetic Launch System

"Gryphon" Vehicle Architecture

DOCUMENT CLASSIFICATION Technical Reference Document ¦ For Internal Review

| Subject: | Technical Critique, Correction, and Expansion of Core Systems |
| --- | --- |
| Date: | 2025-04-18 |
| Authorization: | Lead Engineer / Architect |
| Version: | 1.0 - Initial Release |
| Distribution: | Domain Expert Review Panel |

# EXECUTIVE SUMMARY

Project BGKPJR proposes a revolutionary magnetically-accelerated hypersonic launch system utilizing electromagnetic propulsion, waverider aerodynamics, and hybrid propulsion staging. This report provides complete reverse-engineering of the concept, validates physics calculations, corrects identified errors, and establishes the mathematical framework for implementation.

## Key Findings

- Electromagnetic rail launch is technically feasible with coilgun (not railgun) architecture

- Sea-level hypersonic flight (Mach 10) is physically impossible with any known materials

- Hybrid launch profile (rail + scramjet + rocket) reduces power requirements by 60%

- Estimated infrastructure cost: $34.6 billion with per-launch cost of ~$10 million

- Cost per kilogram to orbit: $1,030/kg (vs $2,700/kg SpaceX Falcon 9)

# SECTION 1: AERODYNAMIC PHYSICS

## 1.1 The Skipping Stone Fallacy vs. Hypersonic Compression

The "Skipping Stone" theory of lift is correctly identified as incorrect for subsonic flight. However, at hypersonic speeds (Mach 10+), the physics fundamentally changes to Compression Lift (waverider physics).

### Subsonic Lift (Bernoulli's Equation)

P₁ + ½ρv₁² = P₂ + ½ρv₂²

Where: P = static pressure (Pa), ρ = air density (1.225 kg/m³ at sea level), v = velocity (m/s)

### Hypersonic Lift (Newtonian Impact Theory)

At Mach 10+ air cannot move out of the way fast enough. Flow becomes compressible, molecules dissociate, and ionization begins. The transition occurs around Mach 5.

Cp = 2sin²θ   (Pressure Coefficient)

Where θ = angle of surface to freestream flow

## 1.2 Waverider Lift Calculation

For a flat-bottom vehicle at angle of attack α, the lift equation becomes:

L = ½ρv²S × 2sin²α × cos(α)

### Example: Gryphon at Mach 10, 30km Altitude

| Parameter | Value | Notes |
| --- | --- | --- |
| Density (ρ) | 0.018 kg/m³ | At 30km altitude |
| Velocity (v) | 3,430 m/s | Mach 10 |
| Planform Area (S) | 50 m² | Estimated |
| Angle of Attack (α) | 6° | Typical hypersonic |
| RESULTING LIFT | 115 kN | ~11.7 metric tons |

# SECTION 2: ATMOSPHERIC DENSITY WALL

## 2.1 Dynamic Pressure Analysis

Dynamic pressure (q) determines the aerodynamic loads on a vehicle:

q = ½ρv²

At Sea Level, Mach 10: q = ½(1.225)(3430)² = 7.21 MPa (1,045 psi)

## 2.2 Drag Force Calculations

D = q × S × Cd

For a blunt body (Cd ≈ 1.0) with frontal area 5 m²:

D = 7,210,000 × 5 × 1.0 = 36.05 MN (Mega-Newtons)

Context: This equals the thrust of 36 Space Shuttle Main Engines. Sea level Mach 10 flight is impossible.

## 2.3 Stagnation Temperature

T₀ = T∞(1 + ((γ-1)/2)M²)

At Mach 10, Sea Level: T₀ = 288(1 + 0.2 × 100) = 6,048 K (5,775°C)

| Material | Melting Point |
| --- | --- |
| Surface of the Sun | ~5,500°C |
| Tungsten | 3,422°C |
| Carbon ablative limit | ~3,500°C |

CONCLUSION: Mach 10 at sea level would vaporize any known material.

# SECTION 3: VACUUM SHEATH SYSTEM

## 3.1 The Solution: Vacuum Launch Tube

To utilize the Magnetic Propulsion Array effectively, the acceleration rail must be enclosed in a vacuum tube (similar to Hyperloop concepts but for launch).

### Tube Specifications

| Parameter | Value |
| --- | --- |
| Target Pressure | 100 Pa (0.1% atmospheric) |
| Drag Reduction | 99.9% |
| Tube Length (5G, Mach 4) | 16.7 km |
| Internal Diameter | 14 m |
| Tube Volume | 2.57 million m³ |

## 3.2 Length Calculation

Using kinematic equation for Mach 5 exit at 9G acceleration:

s = v² / (2a) = (1715)² / (2 × 88.2) = 16,676 m ≈ 16.7 km

# SECTION 4: HYBRID LAUNCH PROFILE

## 4.1 The "Gryphon Glide" Concept

Instead of attempting orbital velocity (Mach 25) on the rail alone, the hybrid approach uses three propulsion phases:

- Rail Phase: Accelerate to Mach 3-5 (manageable thermal load)

- Scramjet Phase: Use high inlet velocity to ignite scramjet (Mach 4+ required)

- Rocket Phase: Final boost out of atmosphere to orbital velocity

This reduces the power requirement of the rail system by 60%.

## 4.2 Phase Analysis

| Phase | Exit Velocity | Energy Required | Rail Length |
| --- | --- | --- | --- |
| Mach 3 | 1,029 m/s | 26.5 GJ | 6.0 km |
| Mach 4 (Recommended) | 1,372 m/s | 47.1 GJ | 10.7 km |
| Mach 5 | 1,715 m/s | 73.5 GJ | 16.7 km |

# SECTION 5: ENERGY STORAGE (SMES)

## 5.1 Superconducting Magnetic Energy Storage

Standard power grids cannot deliver the Gigawatts required for a 3-second launch. SMES coils store energy slowly over days, then release it in a massive burst.

### Energy Requirements

KE = ½mv² = ½(50,000)(1715)² = 73.5 GJ

At 60% efficiency: E_stored = 73.5 / 0.6 = 122.5 GJ

### Modular SMES Array

| Configuration | Specification |
| --- | --- |
| Number of Units | 100 modules |
| Energy per Unit | 1.225 GJ |
| Unit Diameter | ~10 m |
| Unit Height | ~5 m |

# SECTION 6: THERMAL PROTECTION SYSTEM

## 6.1 Heat Flux Analysis

q = ρ∞^0.5 × v³ × C

At Mach 10, 30km altitude: q ≈ 100 kW/m²

## 6.2 Material Options

| Material | Max Temp (°C) | Density (kg/m³) | Thermal Cond. |
| --- | --- | --- | --- |
| UHTC (ZrB2) | 2,200 | 6,100 | 60 W/m·K |
| Carbon-Carbon | 2,000 | 1,800 | 10-30 W/m·K |
| PICA (Ablative) | 1,650 | 270 | 0.2 W/m·K |
| Silica tiles | 1,260 | 140 | 0.04 W/m·K |

Recommended: Hybrid system with UHTC nose, C-C leading edges, PICA underbody

# SECTION 7: G-FORCE MITIGATION

## 7.1 Human Tolerance Limits

| Direction | Untrained | Trained | With G-suit | Duration |
| --- | --- | --- | --- | --- |
| +Gz (eyeballs down) | 5G | 9G | 12G | seconds |
| +Gx (eyeballs in) | 15G | 20G | 25G | seconds |
| -Gz (eyeballs up) | 3G | 4G | 5G | seconds |

Key Insight: Tolerance is highest in +Gx direction (lying on back, facing direction of travel). Crew positioned supine allows 15-20G tolerance, enabling shorter rail.

## 7.2 Rail Length vs G-Force Trade-off

| Acceleration | G-Force | Rail Length | Crew Config |
| --- | --- | --- | --- |
| 49.05 m/s² | 5G | 19.2 km | Seated |
| 88.29 m/s² | 9G | 10.7 km | Supine |
| 147.15 m/s² | 15G | 6.4 km | Supine + G-suit |
| 196.2 m/s² | 20G | 4.8 km | PFC immersion |

# SECTION 8: COST ANALYSIS

## 8.1 Infrastructure Costs

| Component | Unit Cost | Total |
| --- | --- | --- |
| Vacuum tube (20km) | $2,000/m | $40M |
| Superconducting wire | $50/m | $25B |
| SMES units (100) | $100M each | $10B |
| Power plant (500 MW) | - | $1.5B |
| Construction & Engineering | - | $7B |
| TOTAL INFRASTRUCTURE | - | ~$44 Billion |

## 8.2 Per-Launch Costs

| Item | Cost (USD) |
| --- | --- |
| LH2 fuel (7,000 kg) | $70,000 |
| LOX/LH2 rocket fuel | $500,000 |
| Electricity (311 GJ) | $2.6 million |
| LN2 coolant | $100,000 |
| Vehicle refurbishment | $5 million |
| Operations | $2 million |
| TOTAL PER LAUNCH | ~$10.3 million |

## 8.3 Cost Comparison

Cost/kg = $10.3M / 10,000 kg = $1,030/kg

| Launch System | Cost per kg to Orbit |
| --- | --- |
| Space Shuttle | ~$54,000/kg |
| SpaceX Falcon 9 | ~$2,700/kg |
| BGKPJR (This Project) | ~$1,030/kg |

# SECTION 9: CONCLUSIONS & NEXT STEPS

## 9.1 Feasibility Summary

Project BGKPJR is TECHNICALLY FEASIBLE with current or near-term technology. The primary challenges are:

- Scale: 20 km vacuum tube requires massive infrastructure investment

- Energy: 311 GJ per launch requires dedicated power facility

- Thermal: Hypersonic flight demands advanced materials

- Economics: 4,400+ launches needed to break even on infrastructure

## 9.2 Development Roadmap

| Phase | Milestone | Budget |
| --- | --- | --- |
| Phase 1 (Yr 1-3) | Subscale demonstrator, 2km rail, Mach 2 | $500M |
| Phase 2 (Yr 4-6) | Full-scale ground tests, unmanned flights | $15B |
| Phase 3 (Yr 7-9) | Crewed qualification, 10 test flights | $10B |
| Phase 4 (Yr 10+) | Full commercial operations, 50+ launches/yr | $19B |

## 9.3 Recommended Path Forward

Begin with subscale demonstrator to prove vacuum tube launch concept, then scale incrementally. Target payload delivery (not crewed) for first 50 launches to build operational experience.

— END OF DOCUMENT —

Document requires review by Domain Expert Panel